WEEK 3 STATUS REPORT

CCP-CNS2026 | Problem 2: ISP Network Failover

Computer Networks | Instructor: Muhammad Fahad Irshad

| Team Members | Samiya Saeed, Maryam Arshad, Maheera Afif |  |  |
| --- | --- | --- | --- |
| Report Date | 14 June 2026 | Week | Week 3 of 5 |
| Problem | Problem 2: ISP Network Failover |  |  |

1. Week 3 Overview

Week 3 focused on comprehensive testing and validation of the ISP network failover solution. All-to-all ping matrix tests were conducted to verify full connectivity between all devices. Traceroute tests confirmed the correct traffic path before and after link failure. The failover mechanism was tested again to ensure traffic switches to the backup path within 60 seconds.

2. Work Completed This Week

| # | Task | Status | Done By |
| --- | --- | --- | --- |
| 1 | All-to-all ping matrix completed (12 ping tests across all devices) | Completed | Samiya Saeed |
| 2 | Ping from CustomerA to CityA, CityB, CustomerB | Completed | Samiya Saeed |
| 3 | Ping from CustomerB to CityA, CityB, CustomerA | Completed | Samiya Saeed |
| 4 | Ping from CityA to CustomerA, CustomerB, CityB | Completed | Samiya Saeed |
| 5 | Ping from CityB to CustomerA, CustomerB, CityA | Completed | Samiya Saeed |
| 6 | Traceroute test before primary link failure | Completed | Samiya Saeed |
| 7 | Traceroute test after primary link failure (backup path verified) | Completed | Samiya Saeed |
| 8 | Failover timing test (confirmed within 60 seconds) | Completed | Samiya Saeed |
| 9 | All screenshots renamed and uploaded to GitHub Week3 folder | Completed | Maryam Arshad |
| 10 | Week 3 status report prepared | Completed | Maheera Afif |

3. All-to-All Ping Matrix Results

The following table shows ping results between all devices in the network. All tests were conducted with primary link active.

| From \ To | CustomerA 192.168.1.10 | CustomerB 192.168.2.10 | CityA 192.168.1.1 | CityB 192.168.2.1 |
| --- | --- | --- | --- | --- |
| CustomerA (192.168.1.10) | — | ✅ Success | ✅ Success | ✅ Success |
| CustomerB (192.168.2.10) | ✅ Success | — | ✅ Success | ✅ Success |
| CityA (192.168.1.1) | ✅ Success | ✅ Success | — | ✅ Success |
| CityB (192.168.2.1) | ✅ Success | ✅ Success | ✅ Success | — |

4. Traceroute Test Results

Traceroute tests were performed to verify the actual path traffic takes through the network:

| Test | Path Observed | Result |
| --- | --- | --- |
| Traceroute before failure (Primary link UP) | CustomerA → CityA (192.168.1.1) → CityB (10.0.12.2) → CustomerB | Primary path via CityA→CityB direct ✅ |
| Traceroute after failure (Primary link DOWN) | CustomerA → CityA (192.168.1.1) → CityC (10.0.13.2) → CityB → CustomerB | Backup path via CityC confirmed ✅ |

5. Failover Timing Test

The failover timing was measured to confirm the backup path activates within the required 60 seconds:

| Test Parameter | Result |
| --- | --- |
| Primary link shutdown command issued | Interface f0/0 went administratively down immediately |
| Time for backup route to appear in routing table | Less than 5 seconds |
| Ping packets lost during failover | 2-3 packets (normal behavior) |
| Total failover time | Well within 60 second requirement ✅ |
| Primary link restoration time | Immediate when no shutdown applied |

6. Problems Faced & Solutions

| Problem | Solution |
| --- | --- |
| First 2-3 ping packets timeout after failover | Normal behavior — ARP cache needs to update after route change |
| Traceroute showed asterisks on some hops | CityC does not respond to ICMP by default — path still correct |

7. Plan for Week 4

- Perform baseline performance testing using extended ping commands

- Document final GNS3 project version with all configurations

- Create final test matrix with pass/fail results for all tests

- Prepare Week 4 status report

- Review all deliverables and ensure GitHub is up to date

8. GitHub Repository

Repository Link: [Paste your GitHub link here]

Files uploaded this week:

- All ping matrix screenshots (12 screenshots) in Week3/screenshots/

- Traceroute screenshots (before and after failure) in Week3/screenshots/

- Failover test screenshots in Week3/screenshots/

- Week3_Status_Report.pdf

Team Signatures

| Samiya Saeed | Maryam Arshad | Maheera Afif |
| --- | --- | --- |
| Network Architect + Testing | Automation Lead + Security | Documentation + PM |
| Date: 14/06/2026 | Date: 14/06/2026 | Date: 14/06/2026 |