WEEK 2 STATUS REPORT

CCP-CNS2026 | Problem 2: ISP Network Failover

Computer Networks | Instructor: Muhammad Fahad Irshad

| Team Members | Samiya Saeed, Maryam Arshad, Maheera Afif |  |  |
| --- | --- | --- | --- |
| Report Date | 14 June 2026 | Week | Week 2 of 5 |
| Problem | Problem 2: ISP Network Failover |  |  |

1. Project Overview

ConnectSphere ISP operates across three cities: City A (core), City B and City C. This project implements a resilient ISP network using static routing with floating static routes. The primary path from City A to City B uses Administrative Distance (AD) of 1, while the backup path via City C uses AD of 10, ensuring automatic failover within 60 seconds of a link failure.

2. Work Completed This Week

| # | Task | Status | Done By |
| --- | --- | --- | --- |
| 1 | GNS3 topology built with 3 routers (CityA, CityB, CityC) and 2 VPCS | Completed | Samiya Saeed |
| 2 | IP addresses configured on all router interfaces | Completed | Samiya Saeed |
| 3 | Static routes with floating AD configured on CityA and CityB | Completed | Samiya Saeed |
| 4 | CustomerA and CustomerB VPCS configured with IP and gateway | Completed | Samiya Saeed |
| 5 | Ping test from CustomerA to CustomerB successful | Completed | Samiya Saeed |
| 6 | Primary link failure simulated by shutting down CityA f0/0 | Completed | Samiya Saeed |
| 7 | Backup route activation verified via show ip route | Completed | Samiya Saeed |
| 8 | Primary link restored and normal operation confirmed | Completed | Samiya Saeed |
| 9 | Running configs saved for CityA, CityB and CityC | Completed | Maryam Arshad |
| 10 | All files uploaded to GitHub repository | Completed | Maryam Arshad |

3. GNS3 Topology Summary

The following devices were configured in GNS3 this week:

| Device | Type | Role | Key Interface IPs |
| --- | --- | --- | --- |
| CityA | Cisco c3725 | Core ISP Router | f0/0: 10.0.12.1, f0/1: 10.0.13.1, f1/0: 192.168.1.1 |
| CityB | Cisco c3725 | Destination Router | f0/0: 10.0.12.2, f0/1: 10.0.23.2, f1/0: 192.168.2.1 |
| CityC | Cisco c3725 | Backup Hop Router | f0/0: 10.0.13.2, f0/1: 10.0.23.1 |
| CustomerA | VPCS | City A End Device | 192.168.1.10/24 GW: 192.168.1.1 |
| CustomerB | VPCS | City B End Device | 192.168.2.10/24 GW: 192.168.2.1 |

4. Connectivity Test Results

All connectivity tests were performed and screenshots captured as evidence:

| # | Test | Expected Result | Result |
| --- | --- | --- | --- |
| 1 | CustomerA ping 192.168.2.10 (normal operation) | Route via primary path (AD=1) active | PASS |
| 2 | show ip route on CityA before failure | S 192.168.2.0/24 [1/0] via 10.0.12.2 active | PASS |
| 3 | Shutdown f0/0 on CityA to simulate failure | Interface goes administratively down | PASS |
| 4 | show ip route on CityA after failure | S 192.168.2.0/24 [10/0] via 10.0.13.2 active | PASS |
| 5 | Restore f0/0 on CityA with no shutdown | Primary link restored successfully | PASS |

5. Problems Faced & Solutions

| Problem | Solution |
| --- | --- |
| Primary route configured without AD=1 explicitly | Used no ip route to remove old route then re-added with AD=1 |
| Console not opening when double-clicking routers | Configured PuTTY as default console in GNS3 preferences |
| First 2 ping packets timed out after failover | Normal behavior — routing table takes a moment to update after failure |

6. Plan for Week 3

- Implement security features as required by deliverables

- Perform additional failover testing with detailed screenshots

- Document all configurations in final format for GitHub

- Begin drafting Week 3 status report

- Review Week 2 deliverables checklist and ensure all files are on GitHub

7. GitHub Repository

All Week 2 deliverables have been uploaded to the team GitHub repository:

Repository Link: [Paste your GitHub link here]

Files uploaded this week:

- GNS3 project file (project.gns3)

- CityA_config.txt — Running configuration of CityA router

- CityB_config.txt — Running configuration of CityB router

- CityC_config.txt — Running configuration of CityC router

- Ping test screenshots (before failure, after failure, restored)

- Routing table screenshots (show ip route before and after failure)

Team Signatures

| Samiya Saeed | Maryam Arshad | Maheera Afif |
| --- | --- | --- |
| Network Architect + Testing | Automation Lead + Security | Documentation + PM |
| Date: 14/06/2026 | Date: 14/06/2026 | Date: 14/06/2026 |